National Textile University, Faisalabad

Department of Computer Science

Project documentation

ESP32 Smart Radar System( Web Server + Blynk)

Group name :

Yudhra

Group members:

- M.UMAR MUSHTAQ  (23-NTU-CS-1077)

- M.HAMZA FAISAL   (23-NTU-CS-1066)

- M.ABDULLAH   (23-NTU-CS-1056)

1️⃣ INTRODUCTION

In today’s modern security and monitoring systems, object detection and distance measurement play a vital role. Radar-based systems are widely used in ships, submarines, military surveillance, and industrial automation for detecting obstacles and nearby objects.

This project presents an ESP32-based Smart Radar System that uses an ultrasonic sensor and servo motor to perform scanning, detects objects in real time, and displays the results using:

- Web-based Radar Interface (Browser)

- Blynk IoT Mobile Application

- OLED Display & LED Alerts

The system combines embedded systems, IoT, and web technologies to provide real-time visualization and control.

2️⃣ OBJECTIVES OF THE PROJECT

The main objectives of this project are:

- To design a low-cost radar system using ESP32

- To detect nearby objects and measure their distance.

- To visualize radar scanning in real time on a web browser

- To control the system remotely using Blynk mobile app

- To provide visual alerts using LEDs and OLED display

3️⃣ HARDWARE COMPONENTS USED

| Component | Description |
| --- | --- |
| ESP32 | Main microcontroller with Wi-Fi |
| Ultrasonic Sensor (HC-SR04) | Measures object distance |
| Servo Motor | Rotates sensor (0°–180° sweep) |
| OLED Display (SSD1306) | Displays system status |
| Red & Green LEDs | Warning indicators |
| Resistors (220ohm) Jumper wires |  |

4️⃣ SYSTEM ARCHITECTURE

The system is divided into three major layers:

- Sensing Layer

  - Ultrasonic sensor measures distance

  - Servo motor performs radar sweep.

- Processing Layer

  - ESP32 processes sensor data

  - Calculates angle and distance.

  - Controls LEDs and OLED

- Application Layer

  - Web-based radar visualization

  - Blynk mobile app control

BLOCK DIAGRAM :

5️⃣ WORKING PRINCIPLE

- The servo motor rotates the ultrasonic sensor from 0° to 180°.

- At each angle:

  - Ultrasonic sensor sends sound waves.

  - Echo time is measured.

  - Distance is calculated in centimetres.

- ESP32:

  - Updates LEDs based on distance

  - Displays data on OLED.

  - Sends live data to web server.

- Web browser:

  - Shows circular radar.

  - Displays moving sweep line.

  - Shows detected object as red dot.

- Blynk app:

  - Allows ON/OFF control.

  - Enables automation and remote monitoring.

6️⃣ WEB-BASED RADAR SYSTEM

6.1 Web Server Configuration

ESP32 hosts a local web server on port 80 using the Webserver library.

Key endpoints:

- / → Radar UI page

- /toggle → Radar ON/OFF

- /speed → Servo speed control

- /data → JSON data (angle, distance, LED status)

6.2 Radar Web Interface Design

The radar UI is built using:

- HTML

- CSS

- JavaScript

- Canvas API

Radar Features:

- Circular radar screen

- Distance rings

- Cross reference lines

- Green rotating sweep line

- Red dot for detected object

- Live warning messages

6.3 Real-Time Data Communication

ESP32 sends live data in JSON format:

{

"angle": 90,

"dist": 25,

"led": "RED"

}

JavaScript fetches this data every few milliseconds and updates:

- Radar sweep angle

- Object position

- Warning alerts

7️⃣ BLYNK IOT INTEGRATION

7.1 Blynk Platform Overview

Blynk is an IoT platform that allows remote device control using mobile applications.

In this project, Blynk is used to:

- Control radar ON/OFF

- Monitor device online status.

- Enable automation features.

- Show real time distance through chart and numeric value.

7.2 Blynk Functionalities Used

- Virtual buttons for control

- Cloud connectivity

- Real-time device status

- Future expansion for notifications

- Real time distance chart

8️⃣ ALERT & DISPLAY SYSTEM

8.1 LED Alert Logic

| Distance | LED Status |
| --- | --- |
| < 40 cm | Red LED ON |
| ≥ 40 cm | Green LED ON |

8.2 OLED Display Information

OLED shows:

- Radar ON/OFF

- Servo angle

- Distance (cm)

- LED status

- Warning message

9️⃣ RESULTS AND OUTPUT

The system successfully:

- Detects objects accurately.

- Displays radar scan in real time.

- Provides web-based visualization.

- Allows mobile app control.

- Generates alerts for close objects.

FOR NEAR OBJECT VERY CLOSE:

FOR FAR AWAY:

🔟 ADVANTAGES

- Low cost and easy to implement.

- Real-time monitoring

- Remote access via web and mobile

- Expandable for future features

- Educational and industrial use

1️⃣1️⃣ APPLICATIONS

- Ship and submarine navigation (simulation)

- Obstacle detection systems

- Smart surveillance

- Robotics

- Industrial safety monitoring

1️⃣2️⃣ FUTURE ENHANCEMENTS

- Cloud data logging

- Email / push notifications.

- Camera-based object detection

- AI-based object classification

- Mobile radar visualization

1️⃣3️⃣ CONCLUSION

This project successfully demonstrates an ESP32-based smart radar system with real-time web visualization and mobile IoT control. The integration of embedded hardware, web technologies, and IoT platforms makes the system efficient, scalable, and suitable for modern monitoring applications.

🔧 10. SOFTWARE DESIGN & CODE EXPLANATION

10.1 WEB-BASED RADAR SYSTEM – CODE EXPLANATION (WITH KEY CODE)

10.1.1 Libraries Used

Explanation

- WiFi.h
Enables the ESP32 to connect to a wireless network.

- WebServer.h
Allows the ESP32 to act as an HTTP web server.

- ESP32Servo.h
Used to control the servo motor for radar scanning motion.

- Wire.h
Provides I2C communication support for the OLED display.

- Adafruit_GFX.h & Adafruit_SSD1306.h
Used for rendering text and graphics on the OLED screen.

Purpose

These libraries collectively allow sensor interfacing, servo control, web hosting, and OLED visualization within a single embedded system.

10.1.2 Pin Configuration & Hardware Mapping

Explanation

- TRIG_PIN & ECHO_PIN are connected to the ultrasonic sensor.

- SERVO_PIN controls the servo motor using PWM.

- RED_LED & GREEN_LED provide visual warning indications.

Importance

Clear pin mapping ensures reliable communication between the ESP32 and connected hardware components.

10.1.3 Radar Control Variables

Explanation

- radarON controls whether the radar system is active.

- servoAngle stores the current scanning angle.

- direction controls left-to-right or right-to-left sweep.

- stepSize controls the speed of radar scanning.

Radar Logic

The servo motor continuously sweeps from 0° to 180° and back, simulating real radar behavior.

10.1.4 Ultrasonic Distance Measurement Logic

Explanation

- Ultrasonic pulse is transmitted using the TRIG pin.

- Echo time is measured via the ECHO pin.

- Distance is calculated using:

Limitation

Distance is capped at 400 cm to reduce noise and incorrect readings.

10.1.5 LED Warning Logic

Explanation

- Red LED turns ON when an object is closer than 40 cm.

- Green LED remains ON when the distance is safe.

Purpose

Provides instant visual alerts for object proximity.

10.1.6 OLED Display Logic

Explanation

The OLED displays:

- Radar ON/OFF status

- Current servo angle

- Measured distance

- Warning message for close objects

10.1.7 Web Server & API Routes

Route Functions

| Route | Description |
| --- | --- |
| / | Loads radar web interface |
| /toggle | Turns radar ON/OFF |
| /speed | Controls scanning speed |
| /data | Sends live JSON data |

Purpose

Enables real-time data transfer between ESP32 and the web interface.

10.1.8 Radar Web Interface (Frontend Logic)

(JavaScript – Conceptual Overview)

Explanation

- Radar screen drawn using Canvas API

- Rotating sweep line synchronized with servo angle.

- Detected objects shown as red dots.

10.2 BLYNK-BASED RADAR SYSTEM – CODE EXPLANATION

10.2.1 Blynk Configuration

Explanation

Authenticates ESP32 with the Blynk cloud for secure remote access.

10.2.2 Virtual Pins Usage

| Virtual Pin | Function |
| --- | --- |
| V0 | Radar ON/OFF |
| V1 | Servo speed |
| V2 | Angle display |
| V3 | Distance display |

10.2.3 Radar Task Function (Core Logic)

Explanation

This function performs:

- Servo scanning

- Distance calculation

- LED & OLED update

- Live Blynk data transmission

10.2.4 Blynk Timer & Main Loop

Explanation

- Radar task executes every 50 ms

- Ensures smooth scanning and stable cloud connection.

Screenshots of ALL: